AutoStepfinder Auxiliary Tools

This document describes the use of auxiliary tools with ‘’Autostepfinder’’, as described in:

AutoStepfinder: A fast and automated step detection method for single-molecule analysis.

Luuk Loeff, Jacob Kerssemakers, Chirlmin Joo & Cees Dekker.

Last update: November 2020

Contents

Introduction	1

Tools	2

StepMaker:	2

EstimateNoise	2

DeSpiker	3

SlopeMerger	3

ProbeStep_Inject/ ProbeStep_Extract	4

User Notes	5

Few steps	5

Steep (bleach) traces	7

Avoidance of spikes	9

In Conclusion	10

# Introduction

- Tools: A series of useful tools are provided with the main Autostepfinder code. This manual describes how and when to use them.

- User notes: The AutoStepfinder and its default settings are designed to work best for a broad range of applications. However, there may be cases where the default settings of AutoStepfinder need to be fine-tuned to obtain reliable results.  Here we collect some user-cases by means of simple simulated traces, and provide some practical insight in how to tune AutoStepfinder.

# Tools

AutoStepfinder: default analysis package. See main paper for details of use.

## StepMaker:

Description: A compact tool that generates traces for testing. Traces consist of random samplings of user-set step and dwell time distributions as to closely mimick real-life data. Parameters such as step sizes, dwell times, number of steps, noise level and low-pass filtering can be set at will.

For conveniences, a number of example cases are provided. These can easily be changed and expanded in their respective configuration settings.

Use:

- StepMaker('StepTrainDown') runs one standard example:  a step train going down.

  - ‘DownTrendGaussian’

  - ‘DownStepsWithSpikes’

  - ‘FlatDistribution’

  - ‘StepTrainDown’

  - ‘BleachTrace’

  - ‘Custom User ‘

- StepMaker without argument runs all standard examples

Data is saved as text file to use for AutoStepfinder in a sub-directory ‘test_traces’

## EstimateNoise

This function gets a broad estimate of noise levels in a signal.

Approach: pairwise differences are taken between time points to yield a global noise estimate. The distance between time points is varied. A plot of noise level as function of time delay (in points) is shown to the user;

user can indicate a representative noise level

Input:

1) EstimateNoise: no input, runs a demo trace

2) EstimateNoise(1): load data as .tx column file. For more than one column,

we assume column 1 is time and only column 2 is processed.

3) EstimateNoise(trace)

trace is a single or multiple data column as in (2).

Output: user-estimate of noise level

Extras: includes autorun demo option of a low-pass fltered noisy step trace

## DeSpiker

A posteriori removal of spikes  or blinks

Aim: spikes (short up-and-down) or blinks (reverse) may be of less interest to a user. This tool finds and removes these.

Description:

1) DeSpiker: no input, runs a demo trace on trains

4) DeSpiker(spikemaxwidth, updownmargin, spikesign): load data and process according to modus.

with this option, output is saved with extension '_merged'

3) DeSpiker(spikemaxwidth, updownmargin, spikesign,XX,FitX)

Input:

XX is a single data column, FitX is its fit.

spikemaxwidth(5), : max width for spike to consider

updownmargin(0.30): relative magnitude margin per up and down step

spikesign(0):

1: remove positive spikes,

-1: remove negative spikes or 'blinks'

0 remove all signs

Output: new fit and step properties

Extras: includes autorun demo option of a  noisy step trace

## SlopeMerger

Remove slope series by merging

Aim 1: The Autostepfinder tends to fit trains of small steps to smooth

slopes of non-instant steps, for example if the data is low-pass filtered.

This tools fin ds such step trains and merges

them by retaining the middle step index. After that, fit is re-done

Description:

1) SlopeMerger: no input, runs a demo trace

2) SlopeMerger(wmin): load data and process.

with this option, output is saved with extension '_merged'

3) SlopeMerger(wmin,XX,FitX)

Output:

XX is a single data column, FitX is its fit.

wmin(3) is the maximum distance aqual-signed steps should be spaced to

be considered for merging

Output: new fit and step properties

Extras: includes autorun demo option of a  noisy step trace

## ProbeStep_Inject/ ProbeStep_Extract

data_out=ProbeStepInject(spacing,shift, size, data_in)

use:

1) run first this tool; traces are saved in a local directory '\ProbeStepInject\traces'

2  batch-run Autostepfinder on this directory

3) run ProbeStepExtract (see settings there)

description:

ProbeStepInject(stepspacing,stepshift, stepsize, data_in)  uses data

ProbeStepInject(stepspacing,stepshift, stepsize)  user loads data from pwd

ProbeStepInject() makes demo data

parameters:

spacing: the plateau in pts between up-and-down steps.

[note on density]

shift: index where the step series starts.

To boost statistics, shift can be a vector; then multiple saves will be done, each with a shift specified by this vector.

size: up-and-down step size in units of the input data

data_in: single-column data

example: ProbeStepInject(100,1:10:100, 10) user-loads a trace injects a blockwave with

plateaulength 100, amplitude 10, and repeats the action ~10 times with

increasing placement shift.

Output: saves one or multiple traces to be analyzed by AS. As an example,

the first trace is shown.

Extras: includes autorun demo option of a  noisy step trace

# User Notes

Below, some guidelines are given for common situations in analyzing traces

## Few steps

A few steps with many (100) points in the plateaus yields a correct result.

If we now build a curve with very few points per plateau, we observe a tendency to overfit.

(testdata_simulatedDiscreteSteps3Dwell5.txt). Upon inspecting the S-curve, we observe an extremely low secondary peak (right panel, blue curve).

To steer the fit to the far more significant primary fit (the black curve), acceptance threshold can be increased to 1, preventing AutoStepfinder from using the results from round 2:

File: testdata_simulatedDiscreteSteps3Dwell100; generated with StepMaker:

TypeOfSteps='DiscreteSteps';

NumberOfDwells=4;

add_start_or_stop_level=1000;

StepDwellRelation='Dependent';

TypeOfDwells='Exponential';

SignalNoiseRatio=(5)^-1;

Set_distributions line 270:

stepsizes=[-10];  stepcount=[1];  dwelltimes=[100];

Here, inspection of the S-curve offered a guideline how the change the settings (in this case, just increasing the acceptance threshold).

## Steep (bleach) traces

Shown is a simulated, stepped trace with noise, containing 20 equal steps following an exponential decay[1], such as to mimick a bleach curve with simple means.

[1] File: testdata_simulated_BleachTrace20steps; generated with MakeBleachTrace

settings: pts=1000; tau=pts/10; N0=20; noise=0.5;

If we run the AutoStepfinder, it does not detect steps with the default settings:

The AutoStepfinder algorithm is designed to be less sensitive to sloping or drift-like features. In such cases the S-value, as described in the main text, will tend to be very low (in essence, the placement of steps is considered too random).

However, here one may apply user knowledge: if the experiment ensures sloping features should be step like (such as with bleach curves) it is worth enforcing the step fit by lowering the acceptance threshold (here, to 0.05 with all settings kept default):

With a low acceptance threshold, one should inspect the features of the S-curve very carefully. In this case, we observe multiple weak peaks, the largest peaking at only 0.2 with around 20 steps (this one sets the primary fit). We observe that the secondary fit (blue curve) leads to apparent overfitting. However, if we limit the iteration range to 100 steps (which, given that we know we have ‘bleaching events’, is a reasonable choice), we find a fairly reasonable fit:

## Avoidance of spikes

In the following data trace, run was made with ‘advanced options’ ’on’ and ‘S-curves’ on; otherwise default settings. As can be seen, short blinks are also fitted, which occur at a different scale than the steps. In the S-curve at right, we observe a sharp main peak (black line) associated with the main plateaus. After fitting, the remaining spikes have enough significance to yield the blue S-curve with a peak of ~0.3, still above the default of 0.15. Therefore, these steps are fitted in the secondary fit run.

To avoid fitting the blinks, one can set the acceptance threshold higher, to avoid the secondary fit: (note the dotted red line in the right panel, indicating the threshold)

This method will work as long blinks are clearly less significant than the main downward steps. For a constant step size, this implies the duration of the blinks should be much shorter than the ‘regular’ plateaus. Alternatively, one can remove spikes or blinks a posteriori with the ‘DeSpiker’ tool.

Details

Datafile: testdata_simulatedDiscreteSteps20.txt,

Data was generated with Stepmaker with these settings:

NumberOfSteps=20;

add_start_or_stop_level=1000;

StepDwellRelation='InDependent';

TypeOfDwells='Exponential';

SignalNoiseRatio=(5)^-1;

In function ‘’Set_distributions’’ around line 270: ‘’stepsizes=[-10];  stepcount=[1];  dwelltimes=[100];

## In Conclusion

In conclusion, while default settings may not always return an immediate result, a well-guided tuning of the settings (here, just the acceptance threshold and the iteration range) is quite possible. Naturally, such tuning requires some pre-knowledge of the particular experiment (in terms of presence and global number of steps).

A quick and simple trace generator may well provide guidelines how to approach the real data.